國立臺灣師範大學資訊工程學系

九十七學年度第二學期

博士班資格考

考試科目：計算機結構

總分一百分

請在答案卷作答，在題目卷上作答不予計分

- (16分) The following tables list some examples of MIPS machine language formats and its register conventions.

| name | format | example |  |  |  |  |  | comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | R | 0 | 18 | 19 | 17 | 0 | 32 | add $s1,$s2,$s3 |
| sub | R | 0 | 18 | 19 | 17 | 0 | 34 | sub $s1,$s2,$s3 |
| addi | I | 8 | 18 | 17 | 100 |  |  | addi $s1,$s2,100 |
| lw | I | 35 | 18 | 17 | 100 |  |  | lw $s1,100($s2) |
| sw | I | 43 | 18 | 17 | 100 |  |  | sw $s1,100($s2) |
| beq | I | 4 | 17 | 18 | 25 |  |  | beq $s1,$s2,100 |
| bne | I | 5 | 17 | 18 | 25 |  |  | bne $s1,$s2,100 |
| slt | R | 0 | 18 | 19 | 17 | 0 | 42 | slt $s1,$s2,$s3 |
| j | J | 2 | 2500 |  |  |  |  | j 10000 |
| jr | R | 0 | 31 | 0 | 0 | 0 | 8 | jr $ra |
| jal | J | 3 | 2500 |  |  |  |  | jal 10000 |
| Field size |  | 6 bits | 5 bits | 5 bits | 5 bits | 5 bits | 6 bits | All MIPS instructions 32 bits |
| R format | R | op | rs | rt | rd | shamt | funct | Arithmetic instruction |
| I format | I | op | rs | rt | Address/immediate |  |  | Data transfer, branch, and immediate |

| name | Register number |
| --- | --- |
| $zero | 0 |
| $v0-$v1 | 2-3 |
| $a0-$a3 | 4-7 |
| $t0-$t7 | 8-15 |
| $s0-$s7 | 16-23 |
| $t8-$t9 | 24-25 |
| $gp | 28 |
| $sp | 29 |
| $fp | 30 |
| $ra | 31 |

In the memory of a MIPS computer, the contents of locations 80000H to 80007H are listed as follows.

| address | content |
| --- | --- |
| 80000H | 01H |
| 80001H | 04H |
| 80002H | 40H |
| 80003H | 20H |
| 80004H | 20H |
| 80005H | A5H |
| 80006H | FFH |
| 80007H | FFH |

Write down the assembly codes that represent the machine codes stored in these locations.

2. (24分) The following datum represents (a) a signed number, (b) an unsigned number, and (c) a floating point number in IEEE 754 format. What are their corresponding decimal values? You can use sum of exponents to represent the answers.

| bit | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- (10分) Consider two different implementations, M1 and M2, of the same instruction set. M1 has a clock rate of 6GHz, and M2 has a clock rate of 4GHz. There are four classes of instructions (a, B, C, and D) in the instruction set. The average number of cycles for each instruction class on M1 and M2 is listed in the following table:

| Class | CPI on M1 | CPI onM2 |
| --- | --- | --- |
| A | 2 | 1 |
| B | 3 | 2 |
| C | 5 | 3 |
| D | 6 | 4 |

A program is executed on these two machines. The percentage of instruction classes is list as follows.

| Class | Usage |
| --- | --- |
| A | 40% |
| B | 30% |
| C | 20% |
| D | 10% |

Which machine is faster? (You should list all the calculation steps that lead to the conclusion.)

- Consider a 5-stage MIPS pipeline with forwarding.

  - (6分) Use the following code sequence as an example. Explain how the forwarding technique solves the data hazard.

| ADD | R1,R2,R3 |
| --- | --- |
| ADD | R4,R5,R6 |
| SUB | R7,R1,R4 |

  - (6分) Is the forwarding technique always able to solve the data hazard? If your answer is NO, please give an example where the data hazard still occurs even by the employment of forwarding.

- (10分) What is a structural hazard for the pipeline implementation of a CPU? Suppose the instruction memory and data memory are merged to a single memory in the 5-stage MIPS pipeline. That is, assume the pipeline has a single memory instead of two memories. Will we have a structural hazard? Justify your answer.

- (10分) Briefly explain how the branch prediction buffer can be used for dynamic branch prediction.

- Consider two types of caches with the same number of blocks and block size: direct-mapped and fully-associative.

  - (5分) Which type of cache has lower hit time? Justify your answer.

  - (5分) Which type of cache has lower cache miss rate? Justify your answer.

- (8分) Briefly explain why the cache miss penalty can be reduced using the multilevel cache.